A step-by-step Cortex-R validation lab you can run on real hardware

Here is a practical Cortex-R validation lab you can run on real hardware (TI Hercules, NXP S32R, Renesas, etc.).
This is written like a hands-on bring-up + safety validation workflow used in automotive/industrial teams.

🎯 Goal of the lab

Validate that your Cortex-R CPU, memory, interrupts, and safety features work correctly on silicon.

You will verify:

- CPU execution correctness

- TCM/RAM operation

- Interrupt latency

- Timer accuracy

- Exception handling

- Lockstep / safety behavior (if present)

🧰 Required setup

Hardware

- Cortex-R development board

- JTAG debugger

- UART connection

- Oscilloscope or logic analyzer (optional but useful)

Software

- GCC or vendor toolchain

- Debugger (OpenOCD / vendor IDE)

- Simple RTOS (optional) like
FreeRTOS or
Zephyr

You can also run bare-metal.

🧪 LAB 1 — CPU sanity test

Objective

Confirm instruction pipeline and registers work correctly.

Program

int main() {

volatile int a = 5;

volatile int b = 10;

volatile int c = a + b;

while(1);

}

Validate

- Halt CPU in debugger

- Inspect registers

- Confirm c = 15

- Step through instructions

✔ If this fails → clock / pipeline / memory mapping issue

🧪 LAB 2 — TCM / RAM test

Objective

Verify tightly coupled memory or SRAM integrity.

Program

Write a memory test:

#define TEST_ADDR 0x08000000

int main() {

volatile unsigned int *ptr = (unsigned int*)TEST_ADDR;

*ptr = 0xAAAAAAAA;

if (*ptr != 0xAAAAAAAA) while(1);

*ptr = 0x55555555;

if (*ptr != 0x55555555) while(1);

while(1);

}

Validate

- Read memory via debugger

- Check for ECC faults or bus errors

✔ Confirms bus + memory controller + ECC

🧪 LAB 3 — Timer interrupt validation

Objective

Ensure timer interrupts fire correctly.

Steps

- Configure periodic timer (1 ms)

- Toggle GPIO in ISR

void timer_isr() {

GPIO_TOGGLE();

TIMER_CLEAR();

}

Validate

- Measure GPIO on oscilloscope

- Period must match timer value

✔ Confirms interrupt controller + clock tree

🧪 LAB 4 — Interrupt latency test

Objective

Measure real interrupt response time.

Method

- Toggle pin before enabling interrupts

- Toggle inside ISR

- Measure delta with scope

✔ Validates:

- vector table

- interrupt masking

- pipeline flush timing

🧪 LAB 5 — Exception handling test

Objective

Ensure faults are detected safely.

Trigger fault intentionally

volatile int *p = (int*)0xFFFFFFFF;

*p = 5;   // illegal access

Validate

- CPU jumps to data abort handler

- Fault registers populated

- System recovers or logs error

✔ Mandatory for safety certification

🧪 LAB 6 — Context switch test (RTOS)

If using
FreeRTOS

Create two tasks

- Task A toggles LED1

- Task B toggles LED2

Verify:

- Both LEDs blink

- Scheduler switches tasks

- Stack usage stable

✔ Confirms privilege levels + stack protection

🧪 LAB 7 — Lockstep / safety test (if MCU supports it)

Objective

Validate redundant core comparison.

Method

- Inject fault into memory/register

- Observe fault interrupt

- Check safety monitor reaction

✔ Required for ISO 26262 systems

🧪 LAB 8 — Stress test

Run:

- Continuous memory writes

- Interrupt storm

- Timer + DMA + CPU load together

Observe:

- crashes

- missed interrupts

- ECC errors

- watchdog resets

✔ Confirms system robustness

🧪 LAB 9 — Watchdog validation

Test sequence

- Enable watchdog

- Do NOT refresh it

✔ System must reset

Then:

- Refresh periodically

- Ensure system does NOT reset

✔ Confirms safety reset path

📊 Expected final validation report

You should document:

- Boot success rate

- Interrupt jitter

- Timer accuracy

- Memory error detection

- Exception handling correctness

- Watchdog reset time

This becomes your hardware validation baseline.

🚀 If you want, I can also give you:

- A ready-to-run Cortex-R bare-metal test suite

- A GPIO + timer + interrupt demo project

- A Cortex-R safety validation checklist used in automotive companies

- A diagram of Cortex-R exception flow

Tell me which one you want next.