1. Bit Manipulation Questions (Frequently Asked!)

Bit-level operations are fundamental in embedded systems eg. Set Bit, Clear Bit, Toggle Bit, Count Number of Set Bits in a given data etc.

🔄 Q: Reverse the bits of a 32-bit unsigned integer

📌 Why it’s asked: Tests your understanding of bit shifts, performance, and logical operations.

✅ Tips to Prepare:

- Practice operations like rotating bits, counting set bits, finding parity.

- Know how these apply in real-world cases like setting GPIOs or interpreting hardware register flags.

🔁 2. Swapping Arrays

🔄 Q: Swap the contents of two arrays without using a third array

📌 Why it’s asked: Assesses understanding of memory access and data manipulation without additional space.

✅ Tips to Prepare:

- Understand pointer arithmetic.

- Practice in-place array operations: swap, rotate, reverse.

🔤 3. Reverse a String (In-place)

🔄 Q: Reverse a null-terminated string in C

📌 Why it’s asked: Tests handling of strings, memory safety, and edge cases (empty string, one char, etc.).

✅ Tips to Prepare:

- Work with pointers and avoid buffer overruns.

- Be ready to explain memory usage (stack vs heap).

🧠 4. Understanding volatile and const volatile

We see this one a lot in embedded interviews.

❓Q: When do you use volatile?

- For hardware registers

- For shared variables modified in an ISR

- For flags changed by external processes

✅ Prep Tip: Write sample ISR + main loop code that shares a volatile flag.

📚 5. Data Segments & Memory Map Questions

Typical embedded systems questions include:

- What is stored in .bss, .data, .text?

- What happens at startup with these segments?

✅ Prep Tip: Study startup files (startup.s), linker scripts (.ld), and memory maps for your target MCU.

⚙️ 6. Interrupts and ISRs

❓Q: How do you write an ISR in Embedded C? What should be avoided?

- Avoid heavy computation or blocking code in ISRs.

- Declare shared variables as volatile.

✅ Prep Tip: Practice writing GPIO toggle code inside an ISR and configure NVIC or interrupt vectors for your MCU.

💬 Behavioral/Scenario-Based Questions

🔹 How do you debug a hanging embedded system?

🔹 How would you write a driver for a new peripheral?

🔹 What’s your approach when an interrupt isn’t triggering?

✅ Prep Tip: Walk through real experiences — share how you diagnosed an issue using tools like JTAG, oscilloscopes, or by checking register values.

🛠️ Additional Practice Topics

- Endianness: How to detect and convert

- Memory alignment and padding

- Implementing circular buffers

- Writing delay functions using timers (not for loops!)

- Debouncing switches

- State machines in C

✅ Final Preparation Tips

- 🔍 Use simulators or evaluation boards: STM32CubeIDE, Keil uVision, MPLAB X

- 🧪 Code daily: Use platforms like [Coding Ninjas, HackerRank, or LeetCode] (filter for C/Embedded problems)

- 📖 Refer to real manuals: Get used to reading microcontroller datasheets and reference manuals

- 🎯 Mock yourself: Time-box your practice sessions — 45 minutes per coding round is common