Common Types of Microcontroller Resets:

- Power-On Reset (POR): Occurs when power is first applied, clearing registers and starting the program from the beginning to ensure a clean startup.

- Brown-Out Reset (BOR): Triggers when the supply voltage drops below a safe threshold, preventing data corruption and erratic behavior from low voltage.

- External Reset: Activated by an external signal (often a button or supervisor IC) on a dedicated reset pin (like MCLR), pulling the pin low to force a restart.

- Watchdog Timer Reset: An internal timer that, if not periodically reset by software, triggers a reset to recover from software hangs or crashes.

- Software Reset (Internal): Initiated by writing to a specific system register in the firmware, allowing for controlled restarts from within the code.

- Clock Failure Reset: Activates if the internal clock source (oscillator/PLL) malfunctions.

- Core Lockup Reset: Recovers from critical CPU faults or exceptions.

You can see how to use the reset pin on a microcontroller in this video:

1m

gkrsoft

YouTube • 16 Sept 2025

Purpose of Resets:

- Initialization: Sets up hardware (GPIOs, timers, memory) to default states.

- Error Recovery: Brings the system out of hangs, crashes, or unstable conditions.

- System Reliability: Ensures consistent and predictable behavior, especially after power fluctuations or unexpected events.

Implementation:

- Reset Pin: Usually an active-low input, often pulled high with a resistor for normal operation and connected to external circuits.

- RC Delay: A simple resistor-capacitor circuit can delay the reset pin's release, allowing power and clocks to stabilize.

- Reset Cause Register (RCON): Some MCUs have registers to store the reason for the last reset (e.g., BOR, Watchdog) for debugging.