Here are some important Wi-Fi Frequently Asked Questions (FAQ) useful for embedded systems, networking interviews, and IoT development.

1. What is Wi-Fi?

Wi-Fi is a wireless networking technology based on the IEEE 802.11 standard that allows devices to connect to a network using radio waves.

📡 Used in:

- Smartphones

- Laptops

- IoT devices

- Embedded systems (ESP32, Raspberry Pi)

2. What frequency bands does Wi-Fi use?

Wi-Fi operates mainly in:

| Band | Frequency | Features |
| --- | --- | --- |
| 2.4 GHz | 2.4–2.483 GHz | Longer range, more interference |
| 5 GHz | 5.15–5.825 GHz | Faster speed, shorter range |
| 6 GHz | New band for Wi-Fi 6E |  |

3. What are common Wi-Fi standards?

| Standard | Speed | Band |
| --- | --- | --- |
| IEEE 802.11b | 11 Mbps | 2.4 GHz |
| IEEE 802.11g | 54 Mbps | 2.4 GHz |
| IEEE 802.11n | 600 Mbps | 2.4/5 GHz |
| IEEE 802.11ac | ~3.5 Gbps | 5 GHz |
| IEEE 802.11ax | ~9.6 Gbps | 2.4/5/6 GHz |

4. What is an SSID?

SSID (Service Set Identifier) is the name of the Wi-Fi network that users see when scanning networks.

Example:

Home_WiFi
Office_AP
IoT_Network

5. What is BSSID?

BSSID is the MAC address of the Access Point (AP).

Example:

SSID: OfficeWiFi
BSSID: 00:11:22:33:44:55

6. What is an Access Point (AP)?

An Access Point is a device that allows wireless devices to connect to a wired network.

Examples:

- Wi-Fi Router

- Enterprise AP

7. What is the Wi-Fi authentication process?

Wi-Fi connection steps:

- Scanning

- Authentication

- Association

- IP assignment (DHCP)

Important security handshake:

- WPA2 4-Way Handshake

8. What are Wi-Fi security types?

| Security | Description |
| --- | --- |
| Open | No password |
| WEP | Old, insecure |
| WPA | Improved encryption |
| WPA2 | AES encryption |
| WPA3 | Latest secure protocol |

Example protocols:

- WPA2

- WPA3

9. What is RSSI?

RSSI (Received Signal Strength Indicator) indicates signal power.

Example values:

| RSSI | Signal Quality |
| --- | --- |
| -30 dBm | Excellent |
| -50 dBm | Good |
| -70 dBm | Weak |
| -90 dBm | No connection |

10. What is Wi-Fi channel?

Channels divide the frequency band.

Example for 2.4 GHz:

Channel 1
Channel 6
Channel 11

These are commonly used to avoid interference.

11. What is roaming in Wi-Fi?

Roaming allows a device to switch between access points automatically while maintaining the connection.

Used in:

- Office Wi-Fi networks

- Large buildings

12. What is hidden SSID?

A Hidden SSID is a network that does not broadcast its name.
Users must manually enter the SSID to connect.

13. What is Wi-Fi latency?

Latency is the time taken for data to travel between devices.

Typical Wi-Fi latency:

5 ms – 30 ms

14. What is Wi-Fi throughput?

Throughput is the actual data transfer rate achieved in the network.

Example:

Router speed: 300 Mbps
Actual throughput: ~150 Mbps

15. What is the difference between 2.4 GHz and 5 GHz?

| Feature | 2.4 GHz | 5 GHz |
| --- | --- | --- |
| Range | High | Low |
| Speed | Medium | High |
| Interference | High | Low |

Top 30 Wi-Fi Interview Questions for Embedded Engineers

Basic Wi-Fi Concepts

1. What is Wi-Fi?

Wi-Fi is a wireless networking technology based on the IEEE 802.11 standard that allows devices to communicate over radio frequencies.

2. What are the different Wi-Fi standards?

Common standards:

- IEEE 802.11b

- IEEE 802.11g

- IEEE 802.11n

- IEEE 802.11ac

- IEEE 802.11ax

3. What frequency bands are used in Wi-Fi?

Wi-Fi uses:

- 2.4 GHz

- 5 GHz

- 6 GHz (Wi-Fi 6E)

4. What is SSID?

SSID (Service Set Identifier) is the name of the wireless network broadcast by an access point.

Example:

Office_WiFi
HomeNetwork

5. What is BSSID?

BSSID is the MAC address of the Access Point (AP).

Example:

BSSID: 00:11:22:AA:BB:CC

6. What is an Access Point (AP)?

An AP is a device that connects wireless devices to a wired network.

Example:

- Wi-Fi router

- Enterprise AP

7. What is RSSI?

RSSI (Received Signal Strength Indicator) indicates signal strength of Wi-Fi.

Example:

| RSSI | Signal |
| --- | --- |
| -30 dBm | Excellent |
| -60 dBm | Good |
| -80 dBm | Weak |

8. What is Wi-Fi channel?

Channels divide the frequency spectrum for communication.

Example in 2.4 GHz:

Channel 1
Channel 6
Channel 11

Wi-Fi Connection Process

9. What are the steps in Wi-Fi connection?

- Scanning

- Authentication

- Association

- IP address allocation (DHCP)

10. What is active scanning?

In active scanning, the device sends probe request frames to discover networks.

11. What is passive scanning?

In passive scanning, the device listens to beacon frames from access points.

12. What is authentication in Wi-Fi?

Authentication verifies whether the device is allowed to join the network.

13. What is association?

Association is the process where the client registers itself with the Access Point.

Wi-Fi Security

14. What are Wi-Fi security protocols?

- WEP

- WPA

- WPA2

- WPA3

15. What is the 4-way handshake?

The WPA2 4-Way Handshake is used to generate encryption keys between the client and AP.

Steps:

- AP sends ANonce

- Client generates SNonce

- PTK generated

- Secure connection established

16. What encryption is used in WPA2?

WPA2 uses AES encryption.

17. What is WPA3?

WPA3 uses stronger authentication such as SAE (Simultaneous Authentication of Equals).

Embedded / Driver Level Questions

18. What is a beacon frame?

Beacon frames are periodic broadcast messages sent by the Access Point to announce the network.

19. What information is inside a beacon frame?

- SSID

- BSSID

- Channel

- Security type

- Timestamp

20. What is power saving mode in Wi-Fi?

Devices can sleep and wake up periodically to save power.

Used heavily in:

- IoT devices

- Battery powered systems

21. What is Wi-Fi roaming?

Roaming allows a device to switch between multiple access points automatically.

22. What is hidden SSID?

A network where SSID broadcast is disabled.

23. What is throughput vs bandwidth?

Bandwidth
= Maximum theoretical speed

Throughput
= Actual speed achieved.

24. What is Wi-Fi latency?

Latency is the delay between sending and receiving data.

25. What causes Wi-Fi interference?

Common sources:

- Bluetooth

- Microwave ovens

- Other Wi-Fi networks

- Wireless cameras

Advanced Wi-Fi (Important for Embedded/Linux)

26. What is MIMO?

MIMO (Multiple Input Multiple Output) uses multiple antennas to improve throughput.

27. What is OFDM?

OFDM (Orthogonal Frequency Division Multiplexing) splits the signal into multiple sub-carriers.

28. What is a Wi-Fi driver?

A Wi-Fi driver is software that allows the operating system to communicate with the Wi-Fi hardware chip.

Example in Linux:

cfg80211
mac80211

29. What is DHCP in Wi-Fi?

After association, the device obtains an IP address using DHCP protocol.

30. What is the difference between infrastructure mode and ad-hoc mode?

| Mode | Description |
| --- | --- |
| Infrastructure | Uses Access Point |
| Ad-hoc | Device-to-device communication |

Very Important Embedded Wi-Fi Debug Commands (Linux)

Engineers should know these:

ifconfig
iwconfig
iw
iwlist scanning
wpa_supplicant
tcpdump

The complete Wi-Fi packet flow in Linux is very important for embedded Linux, driver development, and networking interviews. It explains how a packet moves from Wi-Fi hardware → driver → kernel → user space application and vice-versa.

The Linux Wi-Fi stack mainly uses mac80211 and cfg80211 frameworks.

1. Linux Wi-Fi Architecture Overview

User Space
   |
   |  (socket API)
   |
Network Stack (TCP/IP)
   |
   |  net_device interface
   |
mac80211 (SoftMAC layer)
   |
cfg80211 (WiFi configuration layer)
   |
Wi-Fi Driver
   |
Wi-Fi Hardware / Firmware

User-space tools that interact with Wi-Fi include:

- wpa_supplicant

- hostapd

2. Packet Flow (Receiving Data)

Example: Laptop receiving a Wi-Fi packet from router

Wi-Fi AP
   ↓
Wi-Fi Radio Hardware
   ↓
Wi-Fi Driver
   ↓
mac80211
   ↓
Linux Network Stack
   ↓
Socket Layer
   ↓
User Application

Step-by-Step Explanation

1. Wi-Fi Hardware

The Wi-Fi chip receives 802.11 frames from the air.

Example chips:

- ESP32

- Qualcomm

- Broadcom

2. Wi-Fi Driver

The driver communicates with hardware via PCIe / SDIO / USB.

Driver tasks:

- Receive interrupt

- Read packet from hardware buffer

- Create sk_buff structure

Example kernel structure:

struct sk_buff

This buffer holds the packet inside Linux.

3. mac80211 Layer

The mac80211 subsystem processes Wi-Fi MAC layer.

Responsibilities:

- 802.11 frame processing

- Decryption

- Reassembly

- QoS handling

It converts:

802.11 frame
        ↓
802.3 Ethernet frame

4. Linux Network Stack

Now the packet enters the Linux networking stack.

Device Driver
   ↓
Net Device Layer
   ↓
IP Layer
   ↓
TCP / UDP

Key protocols handled here:

- IP

- TCP

- UDP

- ICMP

5. Socket Layer

The packet is delivered to the socket buffer used by applications.

Example system calls:

recv()
read()
recvfrom()

6. User Space Application

Finally the packet reaches applications such as:

- Web browser

- MQTT client

- IoT application

- SSH

3. Packet Flow (Sending Data)

Now reverse flow when sending data.

Application
   ↓
Socket API
   ↓
TCP / UDP
   ↓
IP Layer
   ↓
Linux Network Device
   ↓
mac80211
   ↓
Wi-Fi Driver
   ↓
Wi-Fi Hardware
   ↓
Air

Step-by-Step

1. Application Sends Data

Example:

send()
write()
sendto()

2. TCP/IP Stack

Packet gets encapsulated:

Application Data
↓
TCP/UDP Header
↓
IP Header

3. Network Device Layer

The packet is passed to the network interface:

wlan0

4. mac80211 Layer

Adds 802.11 headers and handles:

- encryption

- fragmentation

- aggregation

5. Driver Layer

Driver sends the frame to Wi-Fi hardware.

Communication bus examples:

- PCIe

- SDIO

- USB

6. Wi-Fi Hardware

The hardware converts the frame into RF signals and transmits it.

4. Wi-Fi Control Path (Management)

Configuration commands follow another path.

User Tool
   ↓
cfg80211
   ↓
Driver
   ↓
Hardware

Example tools:

iw
iwconfig
nmcli

These interact with:

- cfg80211

- nl80211

5. Wi-Fi Authentication Flow

When connecting to Wi-Fi:

Application
↓
wpa_supplicant
↓
cfg80211
↓
Driver
↓
Wi-Fi hardware
↓
Access Point

The daemon wpa_supplicant handles:

- WPA2 authentication

- WPA3 authentication

- key management

6. Real Example (ESP32 / Linux IoT Device)

Example packet flow:

MQTT Client
   ↓
TCP
   ↓
IP
   ↓
wlan0 interface
   ↓
mac80211
   ↓
Wi-Fi driver
   ↓
ESP32 Wi-Fi chip
   ↓
Router
   ↓
Internet

7. Important Kernel Structures

Important data structures used in Wi-Fi stack:

| Structure | Purpose |
| --- | --- |
| sk_buff | Packet buffer |
| net_device | Network interface |
| ieee80211_hw | Hardware abstraction |
| ieee80211_vif | Virtual interface |

8. Important Debug Tools

Embedded engineers often use:

iw dev
iw list
iw wlan0 link
tcpdump -i wlan0
dmesg

Packet capture:

tcpdump -i wlan0

ESP32 Wi-Fi connection flow internally

The ESP32 Wi-Fi connection flow internally explains what happens inside the chip when it connects to a router. This is important for IoT, embedded firmware, and driver-level debugging.

The Wi-Fi stack in ESP32 is provided by the ESP-IDF.

1. ESP32 Wi-Fi Software Architecture

Application Layer
      ↓
ESP-IDF Wi-Fi API
      ↓
Wi-Fi Driver
      ↓
Wi-Fi Firmware (MAC + PHY)
      ↓
RF Hardware

Components involved:

| Layer | Role |
| --- | --- |
| Application | Your code |
| ESP-IDF API | Wi-Fi functions |
| Wi-Fi driver | Handles packets |
| Firmware | MAC + PHY control |
| RF | Radio communication |

2. Basic Wi-Fi Connection Code

Typical ESP-IDF Wi-Fi connection sequence:

esp_netif_init();
esp_event_loop_create_default();
esp_netif_create_default_wifi_sta();

wifi_init_config_t cfg = WIFI_INIT_CONFIG_DEFAULT();
esp_wifi_init(&cfg);

esp_wifi_set_mode(WIFI_MODE_STA);

esp_wifi_set_config(WIFI_IF_STA, &wifi_config);

esp_wifi_start();

esp_wifi_connect();

3. Internal Wi-Fi Connection Flow

When ESP32 connects to a router, the following sequence happens.

1 Scan networks
2 Authentication
3 Association
4 WPA handshake
5 IP address assignment

4. Step-by-Step Internal Process

Step 1: Wi-Fi Initialization

Application calls:

esp_wifi_init()

Driver does:

- Allocate Wi-Fi buffers

- Start Wi-Fi task

- Initialize hardware

Step 2: Station Mode Setup

Application configures station mode.

esp_wifi_set_mode(WIFI_MODE_STA)

Modes supported:

| Mode | Description |
| --- | --- |
| STA | Client mode |
| AP | Access point |
| AP+STA | Both |

Step 3: Wi-Fi Scan

ESP32 scans available networks.

esp_wifi_scan_start()

Two types:

| Scan Type | Description |
| --- | --- |
| Active scan | Sends probe requests |
| Passive scan | Listens to beacons |

The access point sends beacon frames.

Step 4: Authentication

ESP32 sends authentication request to the AP.

Authentication types include:

- Open

- WPA2

- WPA3

For WPA2 networks, it uses the WPA2 4-Way Handshake.

Step 5: Association

After authentication:

ESP32 → Association Request
AP → Association Response

Now ESP32 becomes part of the network.

5. WPA2 Handshake Inside ESP32

Key exchange occurs:

AP → ANonce
STA → SNonce
PTK generated
Encryption enabled

Encryption uses:

- AES

6. DHCP Process

After connection, ESP32 requests an IP address.

ESP32 → DHCP Discover
Router → DHCP Offer
ESP32 → DHCP Request
Router → DHCP ACK

The IP stack used is:

- lwIP

7. Packet Data Flow Inside ESP32

Once connected:

Application
   ↓
Socket API
   ↓
lwIP TCP/IP
   ↓
Wi-Fi Driver
   ↓
MAC Layer
   ↓
PHY Layer
   ↓
RF Transmission

8. Important Wi-Fi Tasks Running Inside ESP32

ESP32 internally runs Wi-Fi tasks in FreeRTOS.

Main tasks:

| Task | Purpose |
| --- | --- |
| WiFi task | Manage Wi-Fi driver |
| TCP/IP task | lwIP networking |
| Event task | Wi-Fi events |

9. Important Wi-Fi Events

ESP-IDF generates events like:

WIFI_EVENT_STA_START
WIFI_EVENT_STA_CONNECTED
WIFI_EVENT_STA_DISCONNECTED
IP_EVENT_STA_GOT_IP

Example event handler:

void event_handler(void* arg, esp_event_base_t event_base,
                   int32_t event_id, void* event_data)
{
    if(event_id == WIFI_EVENT_STA_CONNECTED)
    {
        printf("WiFi Connected\n");
    }
}

10. Full ESP32 Wi-Fi Connection Diagram

Application
    ↓
esp_wifi_connect()
    ↓
Wi-Fi Driver
    ↓
Scan Networks
    ↓
Authentication
    ↓
Association
    ↓
WPA2 Handshake
    ↓
DHCP IP Assignment
    ↓
Network Ready

11. Common Wi-Fi Debug Commands (ESP32)

Important debugging functions:

esp_wifi_scan_get_ap_records()
esp_wifi_sta_get_ap_info()
esp_wifi_disconnect()
esp_wifi_connect()

Serial logs help debugging:

idf.py monitor

12. Common Wi-Fi Issues in ESP32

Engineers often debug:

| Problem | Reason |
| --- | --- |
| Connection failure | Wrong password |
| Low RSSI | Weak signal |
| Frequent disconnect | Router issue |
| DHCP timeout | Network issue |

✅ Since you are working with IoT and Raspberry Pi, the next 3 advanced topics will help you a lot in interviews:

Top 20 Wi-Fi Debugging Interview Questions (Automotive / Embedded)

1. Wi-Fi is not connecting to an AP. How will you debug?

Typical debugging steps:

- Check interface

ifconfig -a
ip link show

- Check Wi-Fi scan

iw dev wlan0 scan

- Verify supplicant

ps -ef | grep wpa_supplicant

- Check logs

dmesg | grep wlan
journalctl -u wpa_supplicant

- Check authentication failure.

2. Wi-Fi connected but no internet. How to debug?

Steps:

- Check IP address

ifconfig wlan0

- Check gateway

route -n

- Ping gateway

ping <gateway-ip>

- Check DNS

cat /etc/resolv.conf

- Ping external server

ping 8.8.8.8

3. Wi-Fi scan shows no networks. What could be the issue?

Possible reasons:

- RF kill enabled

- Driver not loaded

- Firmware missing

- Antenna issue

- Regulatory domain mismatch

Debug commands:

rfkill list
lsmod | grep wifi
dmesg | grep firmware

4. How to debug Wi-Fi driver loading failure?

Check:

dmesg | grep wlan

Check firmware:

ls /lib/firmware

Load driver manually:

modprobe <driver_name>

5. How to debug WPA2 authentication failure?

Check logs from:

wpa_supplicant -dd

Look for:

- 4-way handshake failure

- MIC mismatch

- Wrong password

6. How to debug Wi-Fi packet loss?

Check:

ping -i 0.2 <AP>

Use

iw dev wlan0 link

Check:

- RSSI

- Bitrate

- Retry count

Also analyze using Wireshark.

7. How to capture Wi-Fi packets for debugging?

Enable monitor mode.

Example:

iw dev wlan0 set type monitor
tcpdump -i wlan0

Or use Wireshark.

8. How to debug DHCP failure?

Check DHCP client:

dhclient wlan0

Check logs:

journalctl | grep dhcp

Capture packets:

tcpdump -i wlan0 port 67 or port 68

Check DORA process.

9. Wi-Fi connects but disconnects frequently. Why?

Possible reasons:

- Weak signal

- Power saving

- Driver crash

- AP roaming issues

- Interference

Check logs:

dmesg

10. How to debug slow Wi-Fi throughput?

Check:

iw dev wlan0 link

Look for:

- Bitrate

- MCS index

- Channel bandwidth

Use throughput tools:

iperf3

11. How to debug Wi-Fi roaming issues?

Check roaming logs:

wpa_supplicant -dd

Look for:

CTRL-EVENT-ROAMED

Also verify RSSI threshold.

12. How to debug firmware crash in Wi-Fi chipset?

Check kernel logs:

dmesg

Look for:

firmware crashed
recovery triggered

Sometimes firmware dump file generated.

13. How to check regulatory domain issue?

iw reg get

Incorrect regulatory domain can block channels.

Example:

country IN

14. Wi-Fi interface not created after boot. Why?

Check:

- Driver not loaded

- Device tree issue

- PCI/SDIO enumeration failure

Commands:

lspci
lsusb
dmesg

15. How to debug Wi-Fi power management issues?

Check power saving:

iw dev wlan0 get power_save

Disable it:

iw dev wlan0 set power_save off

16. How to debug 4-Way Handshake failure?

Capture packets using Wireshark and check:

Sequence:

1️⃣ Message 1 – AP → STA
2️⃣ Message 2 – STA → AP
3️⃣ Message 3 – AP → STA
4️⃣ Message 4 – STA → AP

Failure occurs if:

- PTK mismatch

- MIC mismatch

17. How to check Wi-Fi driver statistics?

ethtool -S wlan0

or

cat /proc/net/dev

Look for:

- RX errors

- TX retries

- dropped packets

18. How to debug Wi-Fi latency issues?

Use:

ping -i 0.1

Check:

- jitter

- delay spikes

Also analyze:

iperf3 -u

19. How to debug interference in Wi-Fi?

Check channel usage:

iw dev wlan0 scan | grep signal

Use Wi-Fi analyzer tools.

Switch channel.

20. How to debug Wi-Fi driver memory leak?

Check memory usage:

cat /proc/meminfo

Driver debug:

slabtop

Look for growing buffers.

Very Important Automotive Wi-Fi Debugging Tools

Engineers in automotive Linux platforms use:

| Tool | Purpose |
| --- | --- |
| iw | Wi-Fi configuration |
| wpa_supplicant | Authentication |
| tcpdump | Packet capture |
| Wireshark | Packet analysis |
| iperf3 | Throughput testing |
| dmesg | Kernel logs |
| ethtool | Driver statistics |

Real Interview Scenario Question

Example:

"Car infotainment Wi-Fi connects to AP but internet is not working. How will you debug?"

Expected flow:

- Check Wi-Fi link

- Verify IP assignment

- Check gateway

- Check DNS

- Check firewall

- Capture packets